|  |  |  | Hoisting machine @ 18 Tractor Rd |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | CRANE: PALFINGER PK92002SH |  |  |  |  |
|  |  |  |  |  |  |  |  | LORRY CRANE |  |  |  |  |
|  |  |  |  |  |  |  |  | CRANE CONFIGURATI0N |  |  |  |  |
|  |  |  |  |  |  | m6 |  | OPERATING RADIUS: 6m |  |  |  |  |
|  |  |  |  |  |  |  |  | RADIUS (m) |  | LIFTING CAPACITY OF CRANE IN KG |  |  |
|  |  |  |  |  |  |  |  | 4 |  |  | 16,900 |  |
|  |  |  |  |  |  |  |  | 6 |  |  | 12,200 |  |
|  |  |  |  |  |  |  |  | 8 |  |  | 9,100 |  |
|  |  |  |  |  |  |  |  | 10 |  |  | 7,100 |  |
|  |  |  |  |  |  |  |  | 12 |  |  | 5,700 |  |
|  |  |  |  |  | MACHINE WEIGHT: | 3,000 KG |  | SAFETY FACTOR CALCULATION |  |  |  |  |
|  |  |  |  |  |  |  |  | WEIGHT OF CARGO |  |  | : 3, 300 KG |  |
|  |  |  |  |  |  |  |  | LIFTING ACCESSORIES WEIGHT : 50 KG |  |  |  |  |
|  |  |  |  |  |  |  |  | TOTAL WEIGHT OF LIFTING |  |  | : 3,350 KG |  |
|  |  |  |  |  |  |  |  | LIFTING CAPACITY |  |  | :12,000 KG |  |
|  |  |  |  |  |  |  |  | SAFETY FACTOR |  |  | : 3.5 |  |
|  |  |  |  | 6000 |  |  |  |  |  |  |  |  |
|  | CRANE: PALFINGER Pk92002SHH |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DRAWN | APPROVED | DATE | ERIC WONG MACHINERY TRANSPORTATION P/L |  |  |  |
|  |  |  |  |  |  |  | NOK Precision Component (S) Pte Ltd |  | DRAWING NAME: |  |  |  |
|  |  |  |  |  |  |  |  |  | HOISTING OF MACHINE |  |  |  |