|  |  |  | HOISTING OF MACHINE @ 259 Jalan Ahmad Ibrahim |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | CRANE: PALFINGER PK100002F |  |  |  |  |
|  |  |  |  |  |  |  |  | LORRY CRANE |  |  |  |  |
|  |  |  |  |  |  |  |  | CRANE CONFIGURATI0N |  |  |  |  |
|  |  |  |  |  |  | m6 |  | OPERATING RADIUS : 6m |  |  |  |  |
|  |  |  |  |  |  |  |  | OPT . RADIUS(m) |  | LIFTING CAPACITY OF CRANE IN KG |  |  |
|  |  |  |  |  |  |  |  | 4 |  |  | 20, 100 |  |
|  |  |  |  |  |  |  |  | 6 |  |  | 14,300 |  |
|  |  |  |  |  |  |  |  | 8 |  |  | 10,700 |  |
|  |  |  |  |  |  |  |  | 10 |  |  | 8,300 |  |
|  |  |  |  |  |  |  |  | 12 |  |  | 6,600 |  |
|  |  |  |  |  | MACHINE WEIGHT: | 11,000 KG |  | SAFETY FACTOR CALCULATION |  |  |  |  |
|  |  |  |  |  |  |  |  | WEIGHT OF CARGO |  |  | : 11,000KG |  |
|  |  |  |  |  |  |  |  | LIFTING ACCESSORIES WEIGHT : 50 KG |  |  |  |  |
|  |  |  |  |  |  |  |  | TOTAL WEIGHT OF LIFTING |  |  | : 11,050KG |  |
|  |  |  |  |  |  |  |  | LIFTING CAPACITY |  |  | :14,300 KG |  |
|  |  |  |  |  |  |  |  | SAFETY FACTOR |  |  | : 1,29 |  |
|  |  |  |  | 6M |  |  |  |  |  |  |  |  |
|  | CRANE: PALFINGER PK110002SHH |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DRAWN | APPROVED | DATE | ERIC WONG MACHINERY TRANSPORTATION P/L |  |  |  |
|  |  |  |  |  |  |  | VDL |  | DRAWING NAME: |  |  |  |
|  |  |  |  |  |  |  |  |  | HOISTING OF MACHINE |  |  |  |