|  |  |  | Hoisting of machine @ Blk 1091 Lower Delta Rd #01-05 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | CRANE: PALFINGER PK50002EHH |  |  |  |  |
|  |  |  |  |  |  |  |  | LORRY CRANE |  |  |  |  |
|  |  |  |  |  |  |  |  | CRANE CONFIGURATI0N |  |  |  |  |
|  |  |  |  |  |  | m6 |  | OPERATING RADIUS : 6m |  |  |  |  |
|  |  |  |  |  |  |  |  | OPT . RADIUS(m) |  | LIFTING CAPACITY OF CRANE IN KG |  |  |
|  |  |  |  |  |  |  |  | 4 |  |  | 9, 900 |  |
|  |  |  |  |  |  |  |  | 6 |  |  | 7, 000 |  |
|  |  |  |  |  |  |  |  | 8 |  |  | 5, 200 |  |
|  |  |  |  |  |  |  |  | 10 |  |  | 4, 000 |  |
|  |  |  |  |  |  |  |  | 12 |  |  | 3, 250 |  |
|  |  |  |  |  | MACHINE WEIGHT: | 2,100 KG |  | SAFETY FACTOR CALCULATION |  |  |  |  |
|  |  |  |  |  |  |  |  | WEIGHT OF CARGO |  |  | : 2,100 KG |  |
|  |  |  |  |  |  |  |  | LIFTING ACCESSORIES WEIGHT : 50 KG |  |  |  |  |
|  |  |  |  |  |  |  |  | TOTAL WEIGHT OF LIFTING |  |  | : 2, 150 KG |  |
|  |  |  |  |  |  |  |  | LIFTING CAPACITY |  |  | :7, 000 KG |  |
|  |  |  |  |  |  |  |  | SAFETY FACTOR |  |  | : 3.255 |  |
|  |  |  |  | 6M |  |  |  |  |  |  |  |  |
|  | CRANE: PALFINGER PK50002EHH |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DRAWN | APPROVED | DATE | ERIC WONG MACHINERY TRANSPORTATION P/L |  |  |  |
|  |  |  |  |  |  |  | Ming Kee Metal Works PL |  | DRAWING NAME: |  |  |  |
|  |  |  |  |  |  |  |  |  | HOISTING OF MACHINE |  |  |  |