|  |  |  | HOISTING DOWN OF MACHINE AT 3rd FLOOR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | CRANE :PALFINGER PK100002F |  |  |  |  |
|  |  |  |  |  |  |  |  | LORRY CRANE |  |  |  |  |
|  |  |  |  |  |  |  |  | CRANE CONFIGURATI0N |  |  |  |  |
|  |  |  |  |  |  | m6 |  | OPERATING RADIUS : 8m |  |  |  |  |
|  |  |  |  |  |  |  |  | OPT . RADIUS(m) |  | LIFTING CAPACITY OF CRANE IN KG |  |  |
|  |  |  |  |  |  |  |  | 4 |  |  | 16,900 |  |
|  |  |  |  |  |  |  |  | 6 |  |  | 12, 200 |  |
|  |  |  |  |  |  |  |  | 8 |  |  | 9, 100 |  |
|  |  |  |  |  |  |  |  | 10 |  |  | 7, 100 |  |
|  |  |  |  |  |  |  |  | 12 |  |  | 5,700 |  |
|  |  |  |  |  | MACHINE WEIGHT: | 1,600 KG |  | SAFETY FACTOR CALCULATION |  |  |  |  |
|  |  |  |  |  |  |  |  | WEIGHT OF CARGO |  |  | : 1,600 KG |  |
|  |  |  |  |  |  |  |  | LIFTING ACCESSORIES WEIGHT : 50 KG |  |  |  |  |
|  |  |  |  |  |  |  |  | TOTAL WEIGHT OF LIFTING |  |  | : 1, 650 KG |  |
|  |  |  |  |  |  |  |  | LIFTING CAPACITY |  |  | :9, 100 KG |  |
|  |  |  |  |  |  |  |  | SAFETY FACTOR |  |  | : 5, 515 |  |
|  |  |  |  | 8M |  |  |  |  |  |  |  |  |
|  | CRANE : PALFINGER PK100002F |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DRAWN | APPROVED | DATE | ERIC WONG MACHINERY TRANSPORTATION P/L |  |  |  |
|  |  |  |  |  |  |  | Comset Engineering Works |  | DRAWING NAME: |  |  |  |
|  |  |  |  |  |  |  |  |  | HOISTING OF MACHINE |  |  |  |