|  |  |  | HOISTING OF MACHINE AT 2nd FLOOR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | CRANE :PALFINGER PK100002F |  |  |  |  |
|  |  |  |  |  |  |  |  | LORRY CRANE |  |  |  |  |
|  |  |  |  |  |  |  |  | CRANE CONFIGURATI0N |  |  |  |  |
|  |  |  |  |  |  | m6 |  | OPERATING RADIUS : 8m |  |  |  |  |
|  |  |  |  |  |  |  |  | OPT . RADIUS(m) |  | LIFTING CAPACITY OF CRANE IN KG |  |  |
|  |  |  |  |  |  |  |  | 4 |  |  | 17, 840 |  |
|  |  |  |  |  |  |  |  | 6 |  |  | 10, 900 |  |
|  |  |  |  |  |  |  |  | 8 |  |  | 8, 600 |  |
|  |  |  |  |  |  |  |  | 10 |  |  | 7, 000 |  |
|  |  |  |  |  |  |  |  | 12 |  |  | 5,000 |  |
|  |  |  |  |  | MACHINE WEIGHT: | 1,000 KG |  | SAFETY FACTOR CALCULATION |  |  |  |  |
|  |  |  |  |  |  |  |  | WEIGHT OF CARGO |  |  | : 1,000 KG |  |
|  |  |  |  |  |  |  |  | LIFTING ACCESSORIES WEIGHT : 2, 500 KG |  |  |  |  |
|  |  |  |  |  |  |  |  | TOTAL WEIGHT OF LIFTING |  |  | : 3, 500 KG |  |
|  |  |  |  |  |  |  |  | LIFTING CAPACITY |  |  | :8, 600 KG |  |
|  |  |  |  |  |  |  |  | SAFETY FACTOR |  |  | : 2.457 KG |  |
|  |  |  |  | 8M |  |  |  |  |  |  |  |  |
|  | CRANE :PALFINGER PK100002F |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DRAWN | APPROVED | DATE | ERIC WONG MACHINERY TRANSPORTATION P/L |  |  |  |
|  |  |  |  |  |  |  | Energy Press Pte Ltd |  | DRAWING NAME: |  |  |  |
|  |  |  |  |  |  |  |  |  | HOISTING OF MACHINE |  |  |  |