| Eric Wong Machinery Transportation Pte Ltd | PTW / LIFTING PLAN | PTW-LP-01 |  |
| --- | --- | --- | --- |
|  |  | Rev : 00 | Date: 14/09/22 |
|  |  | Page 1 of 2 |  |

| - General |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project: |  |  | Hoisting case onto container |  |  |  |  |  |  |  |  |  |  |
| Location of lifting operation |  |  | 11 Tuas South Ave 12 – Halliburton Completions MFG Pte Ltd |  |  |  |  |  |  |  |  |  |  |
| Date of lifting operation |  |  | 19/09/22 |  |  |  |  | Time of lifting operation: 09:45 am |  |  |  |  |  |
| Validity period of lifting operation: |  |  | 1 day |  |  |  |  |  |  |  |  |  |  |
| - Details of Loads |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of load |  |  | QTN 400M machine |  |  |  |  |  |  |  |  |  |  |
| Overall dimension of load |  |  | 525 x 252 x 210 cm |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  |  | 16, 000 kgs |  |  |  | Known weight |  |  | Estimated weight |  |  |  |
| Centre of gravity: |  |  | Obvious |  |  |  | Estimated |  |  | Determined by drawing |  |  |  |
| - Details of Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Lifting Equipment |  |  |  | Lorry crane |  |  |  |  |  | Mobile crane |  |  |  |
| LM/LE Registration No. |  |  |  | LM474127P |  |  |  |  |  |  |  |  |  |
| Date of last certification |  |  |  | 11/11/21 |  | Expiry date of certificate |  |  |  |  |  |  | 10/11/22 |
| Max. Safe Working Load |  |  |  | 32, 500 kg |  |  |  |  |  |  |  |  |  |
| Max. Boom/Jig length |  |  |  | 43.00 m |  |  |  |  |  |  |  |  |  |
| Intended load radius |  |  |  | 7 m |  |  |  | SWL at this radius |  |  |  | 20, 000 kg |  |
| Type of lifting gears |  |  |  | Wire sling, shackle |  |  |  |  |  |  |  |  |  |
| Combined weight of lifting gears |  |  |  | 200 kg |  |  | SWL of lifting gear |  |  |  |  | 12ton x 4pcs |  |
| Certification of lifting gears |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |
| Expiry date of certificate |  |  |  | 31/07/23 |  |  |  |  |  |  |  |  |  |
| - Means of Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Can the operator see the loading and unloading for the load from his position? |  |  |  |  |  |  | Yes No |  |  |  |  |  |  |
| What at the means of communication between the lifting crew? |  |  |  |  |  |  | Standard hand signals Radio Others_____________________________ |  |  |  |  |  |  |
| - Personnel Involved in Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position |  | Name |  |  |  |  |  |  |  | Qualification / Experience |  |  |  |
| Site Supervisor |  | Koh Ban San/Wong Yen Siong |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Supervisor |  | Koh Ban San/Wong Yen Siong |  |  |  |  |  |  |  | Qualified lifting supervisor |  |  |  |
| Lifting Equipment Operator |  | Yeo Hock Cheng |  |  |  |  |  |  |  | Qualified crane operator |  |  |  |
| Rigger/Signalman |  | Sing Kwok Liang |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |
| Rigger/Signalman |  | Ahmad |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |
| The lifting Supervisor of this lifting operation is: - Name & Title: Koh Ban San / Wong Yen Siong - Signature: |  |  |  |  |  |  |  |  |  | Steps shall be taken to eliminate danger to personnel involved and others including barriers where appropriate |  |  |  |
| - Physical and Environmental Considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ground conditions | Is the ground made safe (eg: placing steel plate) |  |  |  |  |  |  |  |  |  | Yes No |  |  |
|  | Are the outriggers evenly extended? |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Obstacles | Are there any overhead obstacles? |  |  |  |  |  |  |  |  |  | Yes No |  |  |
|  | Are there structure, equipment, stacked materials, etc that obstruct lifting operation from being carried out safely? |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Lighting | Is the lighting condition adequate? |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Demarcation | Has the zone of operation been barricade (with warning signs and barriers) to prevent unauthorised access? |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Other Precautions: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sequence of Operations |  |  |  |  |  |  |  |  | Person in Charge for each step |  |  |  |  |
| 1.Deploy 50ton mobile crane at designated hoisting area |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 2.Attached sling to crane hook and secured onto case rigging point |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 3.Mobile crane to hoist up case 1ft above ground to check on balancing |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 4.When well balanced, hoist up case and load onto container |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 5.Release all sling from case |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 6.Secured case with lashing belt and choking wood |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 7.Once job complete, team leader to instruct team to perform proper housekeeping |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 8.All debris will be clear and dispose |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| 9.Job complete |  |  |  |  |  |  |  |  | Wong Yen Siong / Koh Ban San |  |  |  |  |
| - Approval of Lifting Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied by: Name: Zailani |  |  |  |  | Signature: |  |  |  |  | Date: 19/09/22 |  |  |  |
| Prepared by: Name: Wong Yen Siong |  |  |  |  | Signature: |  |  |  |  | Date: 19/09/22 |  |  |  |
| Assessed by: Name: Kevin Wong |  |  |  |  | Signature: |  |  |  |  | Date: 19/09/22 |  |  |  |
| Approved by: Name: Eric Wong |  |  |  |  | Signature: |  |  |  |  | Date: 19/09/22 |  |  |  |
| This lifting plan shall be prepared by the lifting supervisor. The Department Head responsible shall approve the lifting plan. |  |  |  |  |  |  |  |  |  |  |  |  |  |