| EWMTPL | LIFTING PLAN | LP- MS-01 |  |
| --- | --- | --- | --- |
|  |  | Rev : 00 | Date: 20/03/2020 |
|  |  | Page 1 of 2 |  |

| - General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: 71 Nanyang Drive |  |  |  | Factory: Simtech Valley Block |  |  |  |  |  |  | Process: Moving out / Hoisting machine |  |  |  |  |  |  |
| Date of lifting operation: |  |  | 24/03/2020 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time of lifting operation: |  |  | 09:45 am |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Details of Loads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of load: |  |  |  | Hitachi Seiki VG-45 CNC Milling |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall dimension of load: |  |  |  | 320 x 240 x 250 cm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  |  |  | 3, 000 kgs |  |  |  |  | Known weight |  |  |  | Estimated weight |  |  |  |  |
| Centre of gravity: |  |  |  | Obvious |  |  |  |  | Estimated |  |  |  | Determined by drawing |  |  |  |  |
| - Details of Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Lifting Equipment |  |  |  |  | Lorry crane |  |  |  |  |  | Mobile crane |  |  |  |  |  |  |
| LM/LE Registration No. |  |  |  |  | LM406824P |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of last certification |  |  |  |  | 25/02/2020 |  | Expiry date of certificate |  |  |  |  |  |  |  |  |  | 24/02/2021 |
| Max. Safe Working Load |  |  |  |  | 16, 800 kg |  |  |  |  |  |  |  |  |  |  |  |  |
| Max. Boom/Jig length |  |  |  |  | 22.40 m |  |  |  |  |  |  |  |  |  |  |  |  |
| Intended load radius |  |  |  |  | 6 m |  |  | SWL at this radius |  |  |  |  |  | 12, 200 kg |  |  |  |
| Type of lifting gears |  |  |  |  | Wire sling, shackle |  |  |  |  |  |  |  |  |  |  |  |  |
| Combined weight of lifting gears |  |  |  |  | 50 kg |  |  |  | SWL of lifting gear |  |  |  |  |  | 4ton x 4pcs |  |  |
| Certification of lifting gears |  |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |  |  |  |
| Expiry date of certificate |  |  |  |  | 15/01/2021 |  |  |  |  |  |  |  |  |  |  |  |  |
| - Means of Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Can the operator see the loading and unloading for the load from his position? |  |  |  |  |  |  |  |  | Yes No |  |  |  |  |  |  |  |  |
| What at the means of communication between the lifting crew? |  |  |  |  |  |  |  |  | Standard hand signals Radio Others_____________________________ |  |  |  |  |  |  |  |  |
| - Personnel Involved in Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position |  | Name |  |  |  |  |  |  |  |  | Qualification / Experience |  |  |  |  |  |  |
| Site Supervisor |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Supervisor |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |  | Qualified lifting supervisor |  |  |  |  |  |  |
| Lorry crane Operator |  | Lim Poh Thiam / Norhalim / Ngamin |  |  |  |  |  |  |  |  | Qualified crane operator |  |  |  |  |  |  |
| Rigger/Signalman |  | Rahman/Hassan/Ali/Sing Kwok Liang |  |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |
| Rigger/Signalman |  | Ahmad/Jamari/Hanifah/Asfandi |  |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |
| The lifting Supervisor of this lifting operation is: Name & Title: Ibrahim / Zahari / Koh Ban San / Lifting Supervisor Signature: |  |  |  |  |  |  |  |  |  |  | Steps shall be taken to eliminate danger to personnel involved and others including barriers where appropriate. Tool box meeting to take place and lifting plan be communicated. |  |  |  |  |  |  |
| - Physical and Environmental Considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Floor conditions | Is the floor made safe (eg: placing steel plate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
|  | Are the outriggers evenly extended? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Obstacles | Are there any overhead obstacles? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
|  | Are there structure, equipment, stacked materials that may obstruct lifting operation from being carried out safely? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Lighting | Is the lighting condition adequate? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Demarcation | Has the zone of operation been barricade (with warning signs and barriers) to prevent unauthorised access? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Other Precautions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Lifting Tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sequence of lifting Operations |  |  |  |  |  |  |  |  |  | Person in Charge for each step |  |  |  |  |  |  |  |
| 1.Lift up machine using crowbar |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 2.Insert pallet truck under machine base |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 3.Jack up pallet truck |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 4.Shift out machine to loading bay |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 5.Insert sling onto machine hoisting point and hook onto crane |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 6.Hoist up slowly to check on balancing |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 7.When well balanced, hoist up and load onto lorry chassis |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 8.Release all sling from machine |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 9.Secured machine with strapping belt |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| 10.Move out lorry loader from premise and deliver machine to scrapyard for disposal |  |  |  |  |  |  |  |  |  | Ibrahim / Zahari / Koh Ban San |  |  |  |  |  |  |  |
| - Sketch of Zone of Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sketch of the initial pick-up location, load path and lay-down area (include any obstructions or equipment clashes that may occur and how they will be avoided) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Approval of Lifting Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied by: Name: Zailani |  |  |  |  |  | Signature: |  |  |  |  |  | Date: 24/03/2020 |  |  |  |  |  |
| Prepared by: Name: Ibrahim |  |  |  |  |  | Signature: |  |  |  |  |  | Date: 24/03/2020 |  |  |  |  |  |
| Assessed by: Name: Kevin Wong |  |  |  |  |  | Signature: |  |  |  |  |  | Date: 24/03/2020 |  |  |  |  |  |
| Approved by: Name: Eric Wong |  |  |  |  |  | Signature: |  |  |  |  |  | Date: 24/03/2020 |  |  |  |  |  |
| This lifting plan shall be prepared by the lifting supervisor. The Department Head responsible shall approve the lifting plan. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |