| PAPRDSG | LIFTING PLAN | LP- MS-01 |  |
| --- | --- | --- | --- |
|  |  | Rev : 00 | Date: 18/07/19 |
|  |  | Page 1 of 2 |  |

| - General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: 1 Bedok South Rd |  | Factory: |  |  |  |  |  |  | Process: Hoisting machine |  |  |  |  |  |  |  |
| - Details of Loads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of load: |  | Aida 150ton Single Press |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  | 13, 500 kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall dimensions of load: |  | 150 x 220 x 320 cm |  |  |  |  | Known weight |  |  |  |  | Estimated weight |  |  |  |  |
| Centre of gravity: |  | Obvious |  |  |  |  | Estimated |  |  |  |  | Determined by drawing |  |  |  |  |
| - Details of Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Lifting Equipment |  |  |  | Lorry crane |  |  |  |  | Hoist |  |  |  |  |  |  |  |
| LM/LE Registration No. |  |  |  | LM 084683X |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of last certification |  |  |  | 02/10/2018 |  | Expiry date of certificate |  |  |  |  |  |  |  |  |  | 01/10/2019 |
| Max. Safe Working Load |  |  |  | 18, 560 KG |  |  |  |  |  |  |  |  |  |  |  |  |
| Max. Boom/Jig length |  |  |  | 17.30 m |  |  |  |  |  |  |  |  |  |  |  |  |
| Intended load radius |  |  |  | 5.6 m |  |  |  | SWL at this radius |  |  |  |  | 14,530 kg |  |  |  |
| Type of lifting gears |  |  |  | Wire sling, shackles |  |  |  |  |  |  |  |  |  |  |  |  |
| Combined weight of lifting gears |  |  |  | 100 kg |  |  | SWL of lifting gear |  |  |  |  |  |  | 32 ton |  |  |
| Certification of lifting gears |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |  |  |  |
| Expiry date of certificate |  |  |  | 18/02/2020 |  |  |  |  |  |  |  |  |  |  |  |  |
| - Means of Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Can the operator see the loading and unloading for the load from his position? |  |  |  |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |
| What at the means of communication between the lifting crew? |  |  |  |  |  |  | Standard hand signals Radio Others_____________________________ |  |  |  |  |  |  |  |  |  |
| - Personnel Involved in Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position |  |  | Name |  |  |  |  |  | Qualification / Experience |  |  |  |  |  |  |  |
| Lifting Supervisor |  |  | Wong Yen Siong/Koh Ban San/ Zahari |  |  |  |  |  | Qualified lifting supervisor |  |  |  |  |  |  |  |
| Mobile/Lorry crane Operator |  |  | Koh Soon Hong/ Lim Poh Soon/Lim Poh Thiam |  |  |  |  |  | Qualified crane operator |  |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Sing Kwok Liang |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Asfandi Bin Ja’al |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Ibrahim Bin Mostaman |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Ahmad Bin Abd Rahman |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Md Ali Bin Mahamood |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Abu Hanifah Bin Omar |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |  |
| The lifting Supervisor of this lifting operation is: - Name & Title: Wong Yen Siong / Koh Ban San - Lifting Supervisor - Signature: |  |  |  |  |  |  |  |  | Steps shall be taken to eliminate danger to personnel involved and others including barriers where appropriate. Tool box meeting to take place and lifting plan be communicated. |  |  |  |  |  |  |  |
| - Physical and Environmental Considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Floor conditions | Is the floor made safe (eg: not oily ) |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Obstacles | Are there any overhead obstacles? |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
|  | Are there structure, equipment, stacked materials, etc that obstruct lifting operation from being carried out safely? |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Lighting | Is the lighting condition adequate? |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Demarcation | Has the zone of operation been barricade (with warning signs and barriers) to prevent unauthorised access? |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Other Precautions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Lifting Tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sequence of lifting Operations |  |  |  |  |  |  |  |  |  | Person in Charge for each step |  |  |  |  |  |  |
| 1.Jack up machine to 4” high |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 2.Insert rollers under machine bed |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 3.Using forklift to assist in pulling/pushing machine to loading area |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 4.Using 2 x crane to hoist up and lay down machine |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 5.Hoist up and load machine onto lorry chassis |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 6.Transport to loading area |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 7.Reverse lorry back to back with container |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 8.Hoist up machine and load onto container |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| 9.Secured machine with choking wood and lashing wire |  |  |  |  |  |  |  |  |  | Wong Yen Siong/Koh Ban San |  |  |  |  |  |  |
| - Sketch of Zone of Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sketch of the initial pick-up location, load path and lay-down area (include any obstructions or equipment clashes that may occur and how they will be avoided) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Approval of Lifting Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prepared by: Name: Koh Ban San |  |  |  |  | Signature: |  |  |  |  |  | Date: |  |  |  |  |  |
| Approved by: Name: Eric Wong |  |  |  |  | Signature: |  |  |  |  |  | Date: |  |  |  |  |  |
| This lifting plan shall be prepared by the lifting supervisor. The Department Head responsible shall approve the lifting plan. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Attachments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Attach key critical document / information from the crane or hoist manual to ensure safe lifting operations such as: - Load capacity chart - Range diagram - Rigging method - Sling angle - Etc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |