| EWMTPL | PTW LIFTING PLAN | LP- MS-01 |  |
| --- | --- | --- | --- |
|  |  | Rev : 00 | Date: 11/10/2019 |
|  |  | Page 1 of 2 |  |

| - General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: 51 Marsiling Rd |  | Factory: Beyonics International Pte Ltd |  |  |  |  |  |  | Process: Hoisting machine |  |  |  |  |  |  |
| Date of lifting operation: 19/10/2019 |  |  |  |  |  | Time of lifting operation: 09:45 am |  |  |  |  |  |  |  |  |  |
| - Details of Loads |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of load: |  | 1 unit Aida 300 ton Press Machine |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  | 46, 000 kgs |  |  |  | Known weight |  |  |  |  | Estimated weight |  |  |  |  |
| Overall dimensions of load: |  | 4900 x 3940 x 3500 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Centre of gravity: |  | Obvious |  |  |  | Estimated |  |  |  |  | Determined by drawing |  |  |  |  |
| - Details of Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Lifting Equipment: |  |  |  | Lorry crane |  |  |  |  | Mobile crane |  |  |  |  |  |  |
| LM/LE Registration No: |  |  |  | LM 481691V |  |  |  |  |  |  |  |  |  |  |  |
| Date of last certification: |  |  |  | 13/05/2019 |  | Expiry date of certificate |  |  |  |  |  |  |  |  | 12/05/2020 |
| Max. Safe Working Load: |  |  |  | 79, 700 kg |  |  |  |  |  |  |  |  |  |  |  |
| Max. Boom/Jig length: |  |  |  | 67.80 m |  |  |  |  |  |  |  |  |  |  |  |
| Intended load radius: |  |  |  | 7m |  |  | SWL at this radius |  |  |  |  | 52, 800 kg |  |  |  |
| Type of lifting gears: |  |  |  | Wire sling, shackle |  |  |  |  |  |  |  |  |  |  |  |
| Combined weight of lifting gears: |  |  |  | 300 kg |  | SWL of lifting gear |  |  |  |  |  |  | 15 ton x 4pcs |  |  |
| Certification of lifting gears: |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |  |  |
| Expiry date of certificate: |  |  |  | 02/09/2020 |  |  |  |  |  |  |  |  |  |  |  |
| - Means of Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Can the operator see the loading and unloading for the load from his position? |  |  |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |
| What at the means of communication between the lifting crew? |  |  |  |  |  | Standard hand signals Radio Others_____________________________ |  |  |  |  |  |  |  |  |  |
| - Personnel Involved in Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position |  |  | Name |  |  |  |  |  | Qualification / Experience |  |  |  |  |  |  |
| Lifting Supervisor |  |  | Wong Yen Siong |  |  |  |  |  | Qualified lifting supervisor |  |  |  |  |  |  |
| Mobile / Lorry Crane Operator |  |  | Ong Lai Hock / Lim Poh Soon |  |  |  |  |  | Qualified crane operator |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Sing Kwok Liang / Ibrahim |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |
| Rigger/Signalman |  |  | Asfandi / Ahmad |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |
| Rigger/Signalman |  |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |  |
| The lifting Supervisor of this lifting operation is: - Name & Title: Wong Yen Siong / Lifting Supervisor - Signature: |  |  |  |  |  |  |  |  | Steps shall be taken to eliminate danger to personnel involved and others including barriers where appropriate. Tool box meeting to take place and lifting plan be communicated. |  |  |  |  |  |  |
| - Physical and Environmental Considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Floor conditions | Is the floor made safe (eg: not oily) |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Obstacles | Are there any overhead obstacles? |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
|  | Are there structure, equipment, stacked materials, etc that obstruct lifting operation from being carried out safely? |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Lighting | Is the lighting condition adequate? |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Demarcation | Has the zone of operation been barricade (with warning signs and barriers) to prevent unauthorised access? |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |
| Other Precautions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Lifting Tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sequence of lifting Operations |  |  |  |  |  |  |  | Person in Charge for each step |  |  |  |  |  |  |  |
| 1.Mobile crane to unload machine case from low loader |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 2.Uncrate case |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 3.Insert sling onto machine hoisting point |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 4.Hoist up and upright machine to vertical position |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 5.Load machine onto 1 x cometto |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 6.Release all sling from machine |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 7.Secured machine with strapping belt |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 8.Reverse cometto to loading bay |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 9.Level cometto with loading bay |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 10.Jack up machine and insert rollers under machine bed |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| 11.Shift out machine from cometto and position at designated |  |  |  |  |  |  |  | Wong Yen Siong |  |  |  |  |  |  |  |
| Sketch of Zone of Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sketch of the initial pick-up location, load path and lay-down area (include any obstructions or equipment clashes that may occur and how they will be avoided) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Approval of Lifting Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied by: Name: Zailani |  |  |  |  | Signature: |  |  |  |  | Date: 14/10/19 |  |  |  |  |  |
| Prepared by: Name: Wong Yen Siong |  |  |  |  | Signature: |  |  |  |  | Date: 14/10/19 |  |  |  |  |  |
| Assessed by: Name: Kevin Wong |  |  |  |  | Signature: |  |  |  |  | Date: 14/10/19 |  |  |  |  |  |
| Approved by: Name: Eric Wong |  |  |  |  | Signature: |  |  |  |  | Date: 14/10/19 |  |  |  |  |  |
| This lifting plan shall be prepared by the lifting supervisor. The Department Head responsible shall approve the lifting plan. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |