| Eric Wong Machinery Transportation Pte Ltd | Lifting Plan / Permit to Work | LP/PTW-00 |  |
| --- | --- | --- | --- |
|  |  | Rev: 00 | Date: 28/10/25 |
|  |  | Page 1 of 2 |  |

| - General |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project |  |  | Hoisting of machines at 5th floor through swing gate opening by using platform & mobile crane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Location of lifting operation |  |  | 10 Tampines North Dr #05-04 Distinct Engineering & Trading Pte Ltd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of lifting operation |  |  | 08/11/25 |  |  |  |  |  |  | Time of lifting operation |  |  |  |  |  |  |  | 10:00 am |
| Validity period of lifting operation |  |  | 1 day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Details of Loads to be hoist up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of load: |  |  | 1 unit Micro Dynamics Mega 30V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall dimension of load: |  |  | 270 x 230 x 255 cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  |  | 4, 890 kg |  |  |  | Known weight |  |  |  |  | Estimated weight |  |  |  |  |  |  |
| Centre of gravity: |  |  | Obvious |  |  |  | Estimated |  |  |  |  | Determined by drawing |  |  |  |  |  |  |
| Description of load: |  |  | 1 unit Brother W1000Xd2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall dimension of load: |  |  | 2410 x 2233 x 2633 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  |  | 3, 350 kg |  |  |  | Known weight |  |  |  |  | Estimated weight |  |  |  |  |  |  |
| Centre of gravity: |  |  | Obvious |  |  |  | Estimated |  |  |  |  | Determined by drawing |  |  |  |  |  |  |
| - Details of Load to be hoist down |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description of load: |  | 1 unit GX1000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall dimension of load: |  | 2946 x 2133 x 2717 mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  | 5, 000 kg |  |  |  | Known weight |  |  |  |  | Estimated weight |  |  |  |  |  |  |  |
| Centre of gravity: |  | Obvious |  |  |  | Estimated |  |  |  |  | Determined by drawing |  |  |  |  |  |  |  |
| Description of load: |  | 1 unit VMC600 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall dimension of load: |  | 220 x 190 x 250 cm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weight of load: |  | 5, 000 kg |  |  |  | Known weight |  |  |  |  | Estimated weight |  |  |  |  |  |  |  |
| Centre of gravity: |  | Obvious |  |  |  | Estimated |  |  |  |  | Determined by drawing |  |  |  |  |  |  |  |
| - Details of Lifting Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Lifting Equipment |  |  |  | 601 mobile crane |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LM/LE Registration No. |  |  |  | LM474927J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of last certification |  |  |  | 09/01/25 |  |  |  |  | Expiry date of certificate |  |  |  |  |  |  |  | 07/01/26 |  |
| Max. Safe Working Load |  |  |  | 25, 300 kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max. Boom/Jig length |  |  |  | 37 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Intended load radius |  |  |  | 11 m |  |  |  | SWL at this radius |  |  |  |  |  | 10, 700 kg |  |  |  |  |
| Safety factor |  |  |  | 2.14 |  |  |  | Utilization % |  |  |  |  |  | 46 % |  |  |  |  |
| Type of lifting gears |  |  |  | Wire sling, shackle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Combined weight of lifting gears |  |  |  | 80 kg |  |  | SWL of lifting gear |  |  |  |  |  |  |  | 4ton x 4pcs |  |  |  |
| Certification of lifting gears |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Expiry date of certificate |  |  |  | 26/11/25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Means of Communication |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Can the operator see the loading and unloading for the load from his position? |  |  |  |  |  |  | Yes No |  |  |  |  |  |  |  |  |  |  |  |
| What at the means of communication between the lifting crew? |  |  |  |  |  |  | Standard hand signals Radio Others_____________________________ |  |  |  |  |  |  |  |  |  |  |  |
| - Personnel Involved in Lifting Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position |  |  | Name |  |  |  |  |  |  |  |  |  | Qualification / Experience |  |  |  |  |  |
| Site Supervisor |  |  | Koh Ban San / Zahari / Ibrahim / Wong Yen Siong |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Lifting Supervisor |  |  | Koh Ban San / Zahari / Ibrahim / Wong Yen Siong |  |  |  |  |  |  |  |  |  | Qualified lifting supervisor |  |  |  |  |  |
| Lifting Equipment Operator |  |  | Loh Hock Guan / Norhalim / Ngaimin / Lim Poh Soon |  |  |  |  |  |  |  |  |  | Qualified crane operator |  |  |  |  |  |
| Rigger/Signalman |  |  | Sing Kwok Liang / Rahman / Malik / Aziz |  |  |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |
| Rigger/Signalman |  |  | Ahmad / Rizal / Hanifah / Jamari / Vignesh |  |  |  |  |  |  |  |  |  | Qualified rigger/signalman |  |  |  |  |  |
| - Physical and Environmental Considerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ground condition | Is the ground made safe (eg: placing steel plate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |  |
|  | Are the outriggers evenly extended? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Obstacles | Are there any overhead obstacles? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |  |
|  | Are there structure, equipment, stacked materials, etc that obstruct lifting operation from being carried out safely? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Lighting | Is the lighting condition adequate? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Demarcation | Has the zone of operation been barricade (with warning signs and barriers) to prevent unauthorised access? |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes No |  |  |
| Other Precautions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sequence of lifting Operations: |  |  |  |  |  |  |  |  |  | Person in Charge for each step |  |  |  |  |  |  |  |  |
| 1.Deployment of 2 x mobile crane at designated hoisting area |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 2.Set up crane with outrigger fully extended and resting on steel plate/ timber mats as base plate |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 3.1 x mobile crane to hoist up platform to 5th floor swing gate opening |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 4.Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean against and rested on the opening slab |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 5.Tied back platform to competent point and secured using 1 x mobile crane |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 6.Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 7.Shift out machine from production and load onto platform |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 8.Rigger to insert sling to crane hook and secured to rigging point of machine |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 9.Hoist up machine from platform and lower down to the ground |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 10.Load up machine onto forwarder lorry |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 11.To hoist up new machine to 5th floor, shift and position into location |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| 12.Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose |  |  |  |  |  |  |  |  |  | Ibrahim / Koh Ban San / Zahari / Wong Yen Siong |  |  |  |  |  |  |  |  |
| - Approval of Lifting Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Applied by: Zailani Designation: Supervisor |  |  |  |  | Signature: |  |  |  |  | Date: 28/10/25 |  |  |  |  |  |  |  |  |
| Prepared by: Wong Yen Siong Designation: Lifting Supervisor |  |  |  |  | Signature: |  |  |  |  | Date: 28/10/25 |  |  |  |  |  |  |  |  |
| Assessed by: Kevin Wong Designation: Project Manager |  |  |  |  | Signature: |  |  |  |  | Date: 28/10/25 |  |  |  |  |  |  |  |  |
| Approved by: Eric Wong Designation: Managing Director |  |  |  |  | Signature: |  |  |  |  | Date: 28/10/25 |  |  |  |  |  |  |  |  |
| This lifting plan shall be prepared by the lifting supervisor. The Department Head responsible shall approve the lifting plan / permit to work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |