# METHOD OF STATEMENT

16th July 2024

Description of work

To shift out the machinery from the production area to the loading area, hoist and load onto lorry chassis, and transport to the designated location in accordance with approved lifting procedures and site safety requirements.

Machine Spec

Refer to attached machine list / specifications.

Risk Assessment

A copy of the Risk Assessment shall be attached and briefed to all involved personnel prior to commencement of work.

Date / Time of Operation

24/08/24 – 26/08/24

09:45 am

Location of Operation

19A Serangoon North Ave 5 #01-00, Nichibei Electric Parts (S) Pte Ltd

Equipment Use

4-5 x 633 lorry loader, rollers, pallet truck, hand jack/crowbar, wire/webbing sling, shackle, woodblock

Obstacles

No obstacle on crane parking area and route of moving operation. Barricade operation area to prevent unauthorised entry.

Environment

No operation during heavy rain, thunderstorm or lightning. Maintain good housekeeping and clear debris upon completion.

Lifting Crew

M.O.M. certified lifting supervisor, rigger, signalman and lorry loader operator.

Safety Aspect

Barricade work area with cones, red/white tape and lifting signage. Ensure risk controls are briefed and implemented. All lifting machines/gears to have valid M.O.M. certification. Workers to wear standard PPE.

# Job Scope

1. Conduct site briefing and confirm the approved Risk Assessment and lifting method with all involved personnel.
2. Position the lorry loader at the designated hoisting area and ensure the work area is properly barricaded.
3. Jack up the machine to approximately 3 inches high.
4. Insert rollers and/or pallet truck beneath the machine base.
5. Shift the machine carefully from the production area to the hoisting bay.
6. Rigger to attach the required lifting slings and shackles to the crane hook and secure to the designated rigging points.
7. Carry out a trial lift to check load stability and balance.
8. Upon confirmation that the load is stable, hoist the machine and lower it onto the lorry chassis.
9. Release all lifting gears after the machine is safely landed.
10. Secure the machine with suitable strapping before transportation.
11. Transport the machine to the designated destination.
12. Upon completion, carry out proper housekeeping and remove all debris from the work area.
13. Complete the operation and hand over the area in a safe condition.

Best Regards

Kevin Wong / Zailani