# METHOD OF STATEMENT

Date: 7th November 2025

Description of Work

To unload 4 cases of 2 units machines from 2 x 40ft FR container, uncrate case, shift and position machine into production area at ground floor

Machine Spec

4 Cases of 2 units DA16L20Q Double Column Machining Center

Case 1, 2: 610 x 330 x 375 cm/ Wt: 20,000kgs -each - Main Machine

Case 3,4: 400 x 310 x 220cm – accessories

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate: 10/11/25

Time of Operation

9:00 am

Location of Operation

28 Kranji Loop Kranji Green

Equipment Use

1 x 50ton mobile crane

1-2 x 633 lorry loader

2 x short tail low bed

1 x forklift

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane to near container for the lifting work

- Set up crane with outrigger fully extended

- Insert sling below machine case

- Hoist down cases from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Load machine onto low bed and transport to customer premises

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175