# METHOD OF STATEMENT

Date: 15th Jan 2018

Description of work

1) To unload and hoist down 3 unit machine from lorry chassis, load onto platform, shift and position into premise at ground

2) To shift out 1 unit machine from customer premise, hoist up and load onto forwarder lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18/01/2018   -   09:30 AM

Location of Operation

#### Blk 4012 AMK Ave 10 #01-10

WHITS Technologies

Equipment Use

2 x lorry crane

1 x platform - to lay out for machine pathway

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Lay out platform for machine pathway

- Hoist down machine from lorry chassis and load onto platform

- Insert pallet truck/rollers under machine base

- Shift out machine from platform

- Transport and position machine into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175