# METHOD OF STATEMENT

Date: 30 Sep 2014

Description of work

To delivery 2 unit machine, hoist down and position to location

Machine Spec

1 x Hydraulic Press Brake machine

Dim: 340 x 140 x 270 cm (Wt: 8,500 kgs)

1 x Notching machine

Dim: 910 x 121 x 130 cm (Wt: 780 kgs)

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

02/10/2014 – 09:30 am

Location of Operation

#### No 10 A.M.K Park 2

Daikin Airconditioning (S) Pte Ltd

- Equipment Use

1 x 50 ton lorry crane

Wire sling

*   Obstacles

No obstacle on crane parking/hoisting area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane with load to move in forward/reverse position into inside factory

- Release scrapping belt secured to machine

- Riggers to secured sling to machine hoisting point

- Hoist up machine from lorry chassis

- Lower down machine and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175