# METHOD OF STATEMENT

Date: 04 July 2014

Description of work

To shift out 3 unit machine from workshop to loading area, hoist up onto lorry and deliver to new location

Machine Spec

1 x LCD PPEB 110 Press Brake (Wt: 9,500 kgs)

1 x Hydrabend Bending (Wt: 5,000 kgs)

1 x Notching machine (Wt: 1,000 kgs)

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

To be advised

Location of Operation

#### No 10

Hitachi Chemical (Singapore) Pte Ltd

- Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Heavy duty rollers

Hydraulic jack

8/10 ton wire sling

*   Obstacles

No obstacle on crane parking/hoisting area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman, mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones, red white tape

- Jack up and insert heavy duty rollers to machine base

- Push and shift out machine to loading area

- Hoist up and lay down machine from vertical to horizontal position

- Load up machine onto low-loader

- Machine to be properly secured prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175