# METHOD OF STATEMENT

Date: 15 Jan 2016

Description of work

To shift out 2 unit used 150 Press machine from workshop to loading area, lay down machine and load onto low-loader/lorry

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

22/01/2016 - 09:30am

Location of Operation

#### No 32 Loyang Way

Hitachi Chemical (Singapore) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Heavy duty rollers

Hydraulic jack

Wire sling

Obstacles

No obstacle on crane parking/hoisting area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Barricade operation area with safety cones, red white tape

- Jack up and insert heavy duty rollers to machine base

- Shift out machine from workshop to loading area

- Hoist up and lay down machine to horizontal position

- Load up machine onto low-loader/lorry

- Machine to be properly secured with belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175