# METHOD OF STATEMENT

Date: 21st Feb 2025

Scope of work

To hoist up and unload machine through building rooftop opening by using mobile crane

Machine Spec

1 unit Electrolux Washer: 1565 x 980 x 1610 mm / Wt: 920 kg

1 unit Electrolux Dryer: 1290 x 1295 x 1965 mm / Wt: 447 kg

2 unit Electric Shoe Dryers: 1700 x 600 x 2150 mm / Wt: 350 kg each

1 unit RFID System: 1500 x 1800 x 1800 mm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

05/03/25

Time of Operation

09:30 am

Location of Operation

#### Ebel Industries Pte Ltd

#### 27 Senoko Loop

Equipment Use

1 x 100 ton mobile crane

633 lorry loader

1 set 4ton wire sling

1 set 2/3ton webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

PPE Required

Safety helmet

Safety shoe

Safety Aspect

Barricade off work area with safety cones, red/white and lifting in operation signage

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# Work execution methodology

1. Upon confirming lifting operation, to survey site assessment on the following requirements:

a. Safe access of crane to lifting location

b. Ground stability checks to bring the crane on site

c. Weight of load to be lifted

2. Crane is set up at a distance that is within the SWL of the crane and shall be compensated through the following:

a. Crane operator to check and complete crane pre-hoisting checklist

b. Crane operator to check/verify on manufacture’s manual for SWL and SWR of crane and to be comply with before any hoisting is done

c. Outriggers to be fully extended and resting on metal plate or wooden timber with appropriate dimension as base plate

d. Crane operator to extract the crane boom at the minimum length to compensate the height of   any obstruction during swinging of the boom with load

e. Crane operator shall perform rotational check to confirm no obstruction to crane rotational movement

f. Crane will rotate towards the location and will determine the exact distance and boom angle by lowering the boom until it reach the load location

3. Lifting crew will secure the load to be lifted by ensuring the following:

a. To ensure lifting gears use are in compliance with weight of load to be lifted

b. Riggers to attached all required lifting gears and secure to rigging point of load to be lifted

c. Ensure load to be securely slug, properly balanced and trimmed to prevent from dislodgment

d. Lifting supervisor to check and confirm load are safe to be rig

e. All sharp edges of load shall be padded to protect lifting gears from damage

f. Tagline for load to prevent from swinging so as to prevent bodily contact with load

g. Signalman to communicate to crane operator through walkie-talkie method whenever signalman is not visible to crane operator

Best Regards

Kevin / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175