# METHOD OF STATEMENT

Date: 21st July 2020

Scope of work

To hoist out 1 set Oven through rooftop opening by using mobile crane, load onto low-loader/lorry chassis and deliver to warehouse for packing

Oven spec – to be dismantle by customer engineer

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Date of Operation

TBA

Location of Operation

#### 4 Gul Crescent

AGC Nelco (S) Pte Ltd

Equipment Use

1 x 220 ton mobile crane

2 x 633 lorry loader

1 x low-loader

1 set wire sling

1 set 10ton webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervised by M.O.M qualified lifting supervisor, rigger, signalman and mobile crane operator

# Work execution methodology

1. Upon confirming lifting operation, to survey site assessment on the following requirements:

a. Safe access of crane to lifting location

b. Ground stability checks to bring the crane on site

c. Weight of load to be lifted

d. Identify lifting crew such as lifting supervisor, rigger, signalman and crane operator on site

2. Crane is set up at a distance that is within the SWL of the crane and shall be compensated through the following:

a. Crane operator to check and complete crane pre-hoisting checklist

b. Crane operator to check/verify on manufacture’s manual for SWL and SWR of crane and to be comply with before any hoisting is done

c. Outriggers to be fully extended and resting on base plate or wooden timber with appropriate dimension

d. Crane operator to extract the crane boom at the minimum length to compensate the height of   any obstruction during swinging of the boom with load

e. Crane operator shall perform rotational check to confirm no obstruction to crane rotational movement

f. Crane will rotate towards the location and will determine the exact distance and boom angle by lowering the boom until it reach the load location

3. Lifting crew will secure the load to be lifted by ensuring the following:

a. To ensure lifting gears use are in compliance with weight of load to be lifted

b. Riggers to attached all required lifting gears and secure to rigging point of load to be lifted

c. Ensure load to be securely slug, properly balanced and trimmed to prevent from dislodgment

d. Lifting supervisor to check and confirm load are safe to be rig

e. All sharp edges of load shall be padded to protect lifting gears from damage

f. Tagline for load to prevent from swinging so as to prevent bodily contact with load

g. Signalman to communicate to crane operator through walkie-talkie method whenever signalman is not visible to crane operator

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175