# METHOD OF STATEMENT

Date: 9th July 2014

Description of work

To Hoist 1 unit Kafo CNC Machining Center from 1 x 20ft FR. Uncrate, shift 1 side.

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

09-07-2014

Location of Operation

16 Benoi Sector

- Equipment Use

1 x Kato Crane

Certified Wire Ropes

Heavy Duty Rollers

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc)

Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Kato crane to park at designated area for the lift

- Barricade operation area with safety cones, red white tape

- Lower down machine and insert rollers

- Machine to be properly secured prior to moving off of customer premise

- Workers to shift and position machine to the designated area

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175