# METHOD OF STATEMENT

Date: 14th Sep 2022

Description of work

To shift 1 unit machine from existing location to another location within plant

Machine Spec

1 unit Mazak INTE-E650HII

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

21/09/22

Time of Operation

09:45 am

Location of Operation

#### 2 Gul Circle

#### Cameron (S) Pte Ltd

Equipment Use

1 x lorry loader

1 x 3ton forklift

Hydraulic jack

Woodblock

Electrical roller with control / rollers

Canvas/metal sheet (to lay out for flooring protection)

Obstacles

Ensure no obstacles around operation area and route to new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE

PPE Requirements

Impact Resistant Gloves

Yellow helmet

Coverall

Safety shoes

Safety glass

Job Scope

# Deploy lorry loader at designated location

# Hoist down forklift from lorry chassis

# Fork down equipment needed for the moving from lorry chassis

# To jack up machine using hydraulic jack

# Insert rollers under machine bed

# Shift out machine from existing location

# Transport machine to new location within plant

# Position machine at the designated location

# Unload machine from roller and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175