# METHOD OF STATEMENT

Date: 04th Aug 2023

Description of work

To relocate 1 unit machine from existing location to another location within factory premise

Machine Spec

1 unit Mori Seiki SL-603/2000

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

10/08/23

Time of Operation

09:45 am

Location of Operation

#### 2 Gul Circle

#### Cameron (S) Pte Ltd

Equipment Use

1 x lorry loader

1 x 3ton battery forklift

Hydraulic jack

Woodblock

Rollers

Electrical roller with control

Canvas/metal sheet (to lay out for flooring protection)

Obstacles

Ensure no obstacles around operation area and route to new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety helmet, safety shoe, safety glove)

P.P.E Requirement

Impact Resistant Gloves

Safety helmet

Coverall

Safety shoes

Safety glass

Job Scope

# Arrival of lorry loader onsite and park at designated unloading area

# Lorry loader to hoist down forklift from lorry chassis

# Fork down required work tools for the moving job from lorry chassis

# Insert jack under jacking point of machine

# To jack up machine using hydraulic jack

# Insert rollers under machine bed

# Shift out machine from existing location

# Use of battery forklift to assist in pulling machine

# Transport machine to new location 40m away within factory

# Position machine at the designated location

# Jack up machine to remove rollers

# Insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175