# METHOD OF STATEMENT

Date: 24th Apr 2019

Description of work:

To relocate 1 unit Discovery II D8 CMM machine from 2nd floor to ground floor via cargo lift, shift and position into CMM room

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

29/04/2019

Time of Operation

09:45 am

Location of Operation

5 Tuas Drive 2

Turbine Overhaul Services Pte Ltd

Equipment Use

Pallet truck

1 x battery lifter

Wood block

1 x wooden pallet

Plywood/canvas (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Insert battery lifter fork onto granite table

# Lift up to about 800 mm high

# Remove machine stand away

# Lower down granite table and load onto wooden pallet

# Shift and transfer out machine from 2nd floor to ground floor via cargo lift

# Position machine into CMM room

# Battery lifter to lift up granite table

# Insert stand below

# Lower down granite table and let it sit onto the stand

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175