# METHOD OF STATEMENT

Date: 17th Sep 2018

Description of work:

1) To relocate 1 unit SM100 CNC, 1 unit SPS 20 Excel and 2 unit SLG100 within production area

2) To lift up 1 unit Granite Table onto stand and position within production area at 2nd floor

Risk Assessment

A copy of Risk Assessment will be attached

Start / Completion Date

19/09/2018

Time Of Operation

09:45 am

Location of Operation

No 8 Tractor Rd

Qioptiq Singapore Pte Ltd

Equipment Use

Crowbar

Woodblock

Rollers/pallet truck

1 x battery lifter

Metal plate/canvas (for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# (1)

# Lift up machine base using crowbar

# Insert pallet truck to machine base

# Shift out machine from old location

# Transfer and position machine to new location within production room

# (2)

# Insert webbing sling onto granite table

# Hook sling onto battery lifter fork

# Lift up to about 65cm high

# Adjust machine stand to level with granite table

# Lower down granite table and let it sit onto the stand

# Shift and position within production area

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175