# METHOD OF STATEMENT

Date: 03rd Dec 2018

Description of work:

To relocate 2 unit Makino V22 CNC machine within premise

Risk Assessment

A copy of Risk Assessment will be attached

Start / Completion Date

15/12/2018

Time Of Operation

09:00 am

Location of Operation

Blk 2 Kaki Bukit Ave 1 #01-02

Micro-Mechanics Pte Ltd

Equipment Use

Hand jack/crowbar

Woodblock

Rollers/pallet truck

Metal plate/canvas (for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Lift up machine base using hand jack/crowbar

# Insert pallet truck/rollers to machine base

# Shift out machine from existing location

# Transport and position machine at new designated location within premise

# Unload machine from pallet truck/rollers and insert levelling plate under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175