# METHOD OF STATEMENT

Date: 11th Dec 2020

Description of work:

1) To shift out 1 unit Contour for packing, after packing put aside

2) Using manual stacker to unload 1 unit old Contour from table and put aside, to load 1 unit new Contour onto table

Contour Spec

To be normal pack and put aside

Dim: 80 x 80 x 80 cm

Est Wt: 200-300 kg

To be unload/load onto working table

1 unit old Contour

Wt: 50 kg

1 unit new Contour

Wt: 185 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

14/12/2020 (Monday)

Time of Operation

09:30 am

Location of Operation

27 Senoko South Rd

Heitkamp und Thumann (S) Pte Ltd

Equipment Use

1 x manual stacker

Packing material

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

# Job Scope

# (1)

# Shift out contour from production to loading area

# Using stacker to lift up contour and load onto wooden pallet

# Secured contour onto wooden pallet with strapping belt and choking wood

# Cover with plastic and shrink wrap and put aside

# (2)

# Using manual stacker to unload old contour from working table

# Load contour to ground level and put aside

# Uncrate case for new contour

# Using manual stacker to lift up contour level with working table

# Shift contour slowly from stacker fork onto working table

# Position to location

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175