# METHOD OF STATEMENT

Date: 22nd Nov 2017

Description of work

To relocate 2 unit Toyoda FH-630S and 1 unit Makino S33 machine from one room to another room

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

23/11/2017   -   10:00 am

Location of Operation

#### No 259 Jalan Ahmad Ibrahim

VDL & Enabling Technology Group (S) Pte Ltd

Equipment Use

1 x lorry crane

Manual/hydraulic jack

Woodblock

Rollers/pallet truck

Metal plate (to lay out for epoxy floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Lift up machine using manual/hydraulic jack

# Insert rollers to machine base

# Shift out machine from old location

# Use forklift to assist in pulling/pushing machine

# Shift and position machine to another production room about 30-40m away

# At new location, jack up machine, remove rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and disposed

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175