# METHOD OF STATEMENT

Date: 16th Jan 2018

Description of work:

To relocate 1 unit Lathe and 2 unit Drilling machine within production area

Risk Assessment

A copy of Risk Assessment will be attached

Start / Complete Date

16/01/2018

Time Of Operation

09:45 am

Location of Operation

#### No 259 Jalan Ahmad Ibrahim

VDL & Enabling Technology Group (S) Pte Ltd

Equipment Use

Crowbar/hand jack

Woodblock

Rollers/pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc., safety shoe, safety helmet)

Job scope

# Lift up machine using crowbar/hand jack

# Insert rollers/pallet truck to machine base

# Shift out and transfer machine from old location

# Position machine to new location within production room

# Unload machine from pallet truck/rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175