# METHOD OF STATEMENT

Date: 31st July 2018

Description of work:

To relocate 1 unit Mazak Power Master within production area

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

01/08/2018

Time Of Operation

09:45 am

Location of Operation

#### No 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte Ltd

Equipment Use

Hydraulic/manual jack

Woodblock

Heavy duty rollers

1 x 3 ton battery forklift

Metal sheet/canvas (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc... safety shoe, safety glass)

Job scope

# Jack up machine to 4” using hydraulic/manual jack

# Insert rollers to machine base

# Shift and transfer machine from old location

# Using forklift to assist in pulling/pushing machine

# Position machine to new designated location within production area

# At new location, jack up machine, remove rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Job complete

# Remark:

# To assist engineer dismantle and install back machine cover

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175