# METHOD OF STATEMENT

Date: 20th June 2018

Description of work:

To relocate 1 unit Mazak Slant Turn Nexus 550, 1 unit Mazak Power Master and 1 unit Manipulator to about 40m away within production area

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

25-27/06/2018

Time Of Operation

10:00 am

Location of Operation

#### No 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte Ltd

Equipment Use

Hydraulic jack

Woodblock

Heavy duty rollers

1 x forklift

Metal sheet/canvas (to laid out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc... safety shoe)

Job scope

# Jack up machine to 4” using hydraulic jack

# Insert rollers to machine base

# Shift and transfer machine from old location

# Using forklift to assist in pulling/pushing machine

# Position machine to new designated location within production area

# At new location, jack up machine, remove rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175