# METHOD OF STATEMENT

Date: 07th March 2018

Description of work:

To relocate 1 unit Patec Press, 1 unit Chin Foong Press and 1 unit OCP80N Power Press machine within production area at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

23/03/2018

Time Of Operation

10:00 am

Location of Operation

#### No 33 Gul Lane

Baumann Spring Company (S) Pte Ltd

Equipment Use

Hydraulic jack

Woodblock

Heavy duty rollers

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc... safety shoe)

Job scope

# Jack up machine to 4” using hydraulic jack

# Insert rollers to machine base

# Shift and transfer machine from old location

# Using forklift to assist in pulling/pushing machine

# Position machine to new designated location within production area

# At new location, jack up machine, remove rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175