# METHOD OF STATEMENT

Date: 03rd July 2019

Description of work:

To relocate 6 unit machine within customer production area

Machine Spec

1 unit Warner & Swasey SC-25 CNC Lathe

2 unit Doosan Puma HM800

1 unit W & S SC-36 CNC Lathe

1 unit Doosan Puma 800XL

1 unit Mazak SL-60

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

As per required schedule

Time Of Operation

09:45 am

Location of Operation

#### 80 Tuas West Drive

Dril-Quip Asia Pacific Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Heavy duty rollers/pallet truck

1 x 3 ton battery forklift

Canvas/metal plate (to protect epoxy floor)

Obstacles

To clear all obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE

Job scope

# Jack up machine to 4” using hydraulic/hand jack

# Insert rollers to machine base

# Shift out and transfer machine to new location

# Position machine at new designated location within production area

# Jack up machine, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175