# METHOD OF STATEMENT

Date: 27th July 2017

Description of work:

Phase 1

1 unit Haas Mini Milling – From Building E1 to Building C1

2 unit Milling – Shift within Building C1

1 unit Enshu Milling – Shift within Building C1

Phase 2

Relocate 5 unit 6S machine within Building E1

Phase 3

Shift out from Building E1 and transfer to F Lobby

1 unit Mini Milling

1 unit Haas TL-1 NC Lathe

1 unit TAL-460 Manual Lathe

1 unit Bandsaw BS-250HS

Risk Assessment

A copy of Risk Assessment will be attached

Start / Completion Date

As per required schedule

Time Of Operation

10:00 am

Location of Operation

#### No 91 Tuas Ave 1

SMC Mfg (S) Pte Ltd

Equipment Use

Crowbar

Manual jack

Woodblock

Metal plate/plywood (for epoxy floor protection)

Rollers/pallet truck

1 x battery forklift

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc., safety shoe, safety helmet)

# Job Scope

# Lift up machine base using crowbar/manual jack

# Insert rollers/pallet truck to machine base

# Shift out machine from old location

# Transfer and position machine at new location as per arrangement

# Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175