# METHOD OF STATEMENT

Date: 28th Feb 2018

Description of work:

To relocate 2 unit Toyoda FH 63S CNC machine @ estimate weight of 16 ton each to new location within production area at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Start / Complete Date

06/03/2018   -   06/03/2018

Time Of Operation

09:30 am

Location of Operation

#### No 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Canvas/metal sheet (for epoxy floor protection)

1 x battery forklift

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc., safety shoe)

Job scope

# Jack up machine to 4” using hand/hydraulic jack

# Insert rollers to machine base

# Lay out canvas/metal sheet to protect flooring

# Shift and transfer machine from old location

# Using forklift to assist in pulling/pushing machine

# Position machine to new designated location within production area

# At new location, jack up machine, remove rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Job complete

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175