# METHOD OF STATEMENT

Date: 28th Sep 2017

Description of work

To relocate 1 unit VMX-50/50T CNC machine from Bay 6 to Bay 8

Date / Time of Operation

04/10/2017   -   10:00 am

Location of Operation

#### No 2 Pioneer Sector 3

GE Keppel Energy Services Pte Ltd

Equipment Use

1 x lorry crane

Manual/hydraulic jack

Woodblock

Rollers

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine position

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Lift up machine using manual/hydraulic jack

# Insert rollers to machine base

# Shift out machine from production at bay 6

# Use forklift to assist in pulling machine and transfer to bay 8

# At bay 8, shift and position machine about 40m to the designated location

# Jack up machine, remove rollers and let it sit on machine plate

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and disposed

# Remark:

# MOS subject to change by supervisor on site

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175