# METHOD OF STATEMENT

Date: 15th Aug 2019

Description of work

To relocate and shift out 1 unit machine from production are to outside factory open space for temporary storage

Machine Spec

1 unit Bullard 42 VTL

Dim: 300 x 280 x 310 cm

Wt: 10, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

16/08/2019   -   09:30 am

Location of Operation

#### No 23 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers

Canvas (to cover up machine at storage area)

Obstacles

No obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Jack up machine to 3” high

# Insert rollers under to machine bed

# Shift out machine from production area to outside open space

# Position machine at temporary location for storage

# Unload machine from rollers and insert woodblock under machine bed

# Cover up machine with canvas and shrink wrap

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175