# METHOD OF STATEMENT

Date: 01st Sep 2022

Description of work

To shift 1 unit machine a few meter away from existing location to do marking and drill hole, shift and position back to location after drilling

Machine Spec

1 unit Mazak IG400

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12-13/09/22

Time of Operation

09:45 am

Location of Operation

#### 11 Tuas South Ave 12

#### Halliburton Completions MFG Pte Ltd

Equipment Use

Hydraulic jack

Woodblock

Air rollers

Hand drill

1 x 3ton battery forklift

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Jack up machine using hydraulic jack

# Insert air rollers under machine bed

# Shift machine from existing location to a few meter away

# Mark and drill hole for low plug

# After drill hole, to shift back machine

# Position machine at the designated location

# Unload machine from roller and let it sit on level ground

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175