# METHOD OF STATEMENT

Date: 11th June 2021

Description of work

To relocate 1 unit machine from existing location to new location within premise

Machine Spec

1 unit CNC Polishing machine

Dim: 250 x 200 x 220 cm

Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

14/06/21

Time Of Operation

09:45 am

Location of Operation

#### 209 Woodlands Sector 2 #02-58

Equipment Use

Crowbar

Woodblock

1 x 5ton pallet truck

Metal sheet/canvas (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

Job Scope

# Insert pallet truck under machine bed

# Shift out machine from existing location

# Transport machine to new location within production

# Position machine at the designated location

# Unload from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris, if any will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175