# METHOD OF STATEMENT

Date: 02nd June 2022

Description of work

To shift out 1 unit generator from existing location to building basement

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

Equipment Use

Hydraulic jack

Hydraulic rollers

Solid woodblock

Obstacles

Ensure no obstacles around operation area and route to new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard P.P.E (safety shoe, safety helmet)

Job Scope

# Jack up one side of generator using hydraulic jack

# Lower down and let it sit onto solid woodblock

# Jack up another side and insert hydraulic roller under generator bed

# Roller to be secure with bolts and clamping plate to avoid generator skid away from roller

# Shift out generator from existing location

# Transport to building basement

# Position at the new designated location

# Unload generator from roller and let it sit on level ground

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175