# METHOD OF STATEMENT

Date: 05th Sep 2023

Description of work

To relocate ATC within factory premise

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

05/09/23

Time of Operation

09:45 am

Location of Operation

#### 16E Tuas Ave 1 #01-61

Hunting Energy Services Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Jack up ATC to 4” high

# Insert rollers under ATC bed

# Shift out ATC from existing location

# Transport to new location within factory premise

# Position ATC at designated location

# Jack up ATC, insert woodblock and remove rollers

# Jack up another side of ATC, remove rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Any debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175