# METHOD OF STATEMENT

Date: 24th Sep 2020

Description of work

To relocate machine within customer factory premise

Machine Spec

1 unit Mazak E-Tower 1060/8 – Shift out from Bay 2 to Bay 1

1 unit Toshiba TUD15 – Shift 6m away

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/09/2020

Time of Operation

09:45 am

Location of Operation

#### 127 Pioneer Rd

Fong Lee Metal Industries Pte Ltd

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Job Scope

# Jack up machine using hydraulic jack

# Insert rollers under machine base

# Shift out machine from old location

# Transport machine to new location within production area

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175