# METHOD OF STATEMENT

Date: 11th Feb 2025

Description of work

To relocate 3 unit equipment from workshop to open space area outside workshop

Machine Spec

2 unit Gearbox

Dim: 180 x 160 x 220 cm / Wt: 7, 000 kg each

1 unit small diesel engine

Dim: 170 x 110 x 155 cm / Wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/02/25

Time of Operation

09:45 am

Location of Operation

#### 2 A.M.K Drive

ITE College Central

Equipment Use

1 x 633 lorry loader

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Wire sling/shackle

Obstacles

Ensure no obstacles around operation area and route to machine new location

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involve in this operation

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Lift up equipment using hydraulic/hand jack

# Insert rollers under equipment base

# Shift out equipment from workshop to outside

# Riggers to insert sling to crane hook

# Secured sling to rigging point of equipment

# Hoist up equipment and load onto lorry chassis

# Transfer to open space with workshop area

# Hoist down equipment from lorry chassis

# Position equipment within open space area

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175