# METHOD OF STATEMENT

Date: 11th Jan 2022

Description of work

To relocate 1 unit machine within factory premise

Machine Spec

1 unit Citizen L20

Wt: 2, 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/01/22

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

#### Panasonic Factory Solutions Asia Pacific

Equipment Use

Crowbar

Woodblock

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety helmet)

# Job Scope

# Lift up machine using crowbar

# Insert pallet truck under machine base

# Shift out machine from existing location

# Transport machine to new location within factory

# Position machine at the designated location

# Unload machine from pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175