# METHOD OF STATEMENT

Date: 16th May 2019

Description of work

To relocate 1 unit Kent KGS-63AHD Grinder machine within factory premise

Risk Assessment

A copy of Risk Assessment will be attached

Date Of Operation

21/05/2019

Time Of Operation

09:45 am

Location of Operation

#### 2 Pioneer Sector 3

GE Repair Solutions (S) Pte Ltd

Equipment Use

Crowbar

Woodblock

Pallet truck

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

# Job Scope

# Lift up machine using crowbar

# Insert pallet truck under machine base

# Shift out machine from old location

# Transport machine to new location within production area

# Position machine at the designated location

# Unload machine and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175