# METHOD OF STATEMENT

Date: 28th Feb 2024

Description of work

To relocate following machines from blk 16 to blk 20

Machine Spec

3 sets Hand Type Bending Machine (Wt: 150kgs-each)

2 sets Lathe Machine (Wt: 750kgs-each)

2sets 3HP Milling Machine (Wt: 1,250kgs-each)

2 sets Horizontal Milling Machine (Wt: 1,600kgs-each)

1 set Vertical Bandsaw (Wt; 300kgs)

1 set Horizontal Bandsaw (Wt: 320kgs)

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

08/03/24

Time of Operation

10.30am

Location of Operation

#### Temasek Polytechnic

#### 21 Tampine Ave 1, Blk 16 & Transfer to Blk 20

Equipment Use

1 x 633 lorry loader

Crowbar

Woodblock

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Lift up machine using crowbar jack

# Insert rollers/pallet truck under machine base

# Shift out machine from blk 16 to loading area

# Insert sling to crane hook and secured to rigging point of machine

# Hoist up machine and load onto lorry chassis

# Transfer machine to block 20

# Hoist down machine from lorry chassis to near ground level

# Insert rollers/pallet truck under machine bed

# Shift and transport machine into factory premise

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175