# METHOD OF STATEMENT

Date: 11th Dec 2024

Description of work

To relocate 2 unit machines within factory premise

Machine Spec

1 unit Sunnen Deep Hole Drilling / Wt: 1, 000 kg

1 unit Non Ferrous Oven / Wt: 4, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

19/12/24

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes (S) Pte

Equipment Use

1 x 633 lorry loader

Hand jack / crowbar

Woodblock

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glove)

Job Scope

# Lift up machine using crowbar/hand jack

# Insert rollers/pallet truck under machine base

# Shift out machine from existing location

# Turn machine to 180degree

# Position machine at the new designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175