# METHOD OF STATEMENT

Date: 28th Aug 2020

Description of work

To relocate 1 unit machine within factory premise

Machine Spec

1 Unit Torque Machine

Machine Weight: 10 Tons

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/09/2020

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes (S) Pte

Equipment Use

Crowbar / Hand Jack

Woodblock

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Job Scope

# -Lift up machine using crowbar / hand jack

# -Insert pallet truck under machine base

# -Shift out machine from old location

# -Transport machine to new location within production area

# -Position machine at the designated location

# -Unload machine from pallet truck and insert levelling pad under machine bed

# -Once job complete, team leader will instruct the team to perform proper housekeeping

# -All debris will be clear

# -Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175