# METHOD OF STATEMENT

Date: 04th May 2023

Description of work

To relocate 3 unit machines within factory premise

Machine Spec

1 unit Sunnen Horizontal Hone

1 unit long Oven

1 unit Shear Test machine

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/05/23

Long Oven (from bay 6 to bay 6)

Shear Test machine (bay 3 to bay 3)

Sunnen Horizontal Hone (to shift out from bay 6 to loading bay, hoist up onto lorry chassis and transfer to bay 3)

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes (S) Pte

Equipment Use

1 x 633 lorry loader

Hand jack / crowbar

Woodblock

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

1 x forklift

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe, safety glass)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

Job Scope

# Lift up machine using crowbar/hand jack

# Insert rollers/pallet truck under machine base

# Shift out machine from old bay to loading area

# Insert sling to crane hook and secured to rigging point of machine

# Hoist up machine and load onto lorry chassis

# Transfer machine from bay 6 to bay 3

# Hoist down machine from lorry chassis to near ground level

# Insert rollers under machine bed

# Shift and transport machine into factory premise

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175