# METHOD OF STATEMENT

Date: 11th Mar 2020

Description of work

To relocate 1 unit machine within production area at ground floor

Machine Spec

1 unit Matsuura H Plus-300 PC-11

Dim: 380 x 250 x 250 cm

Wt: 9, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/03/2020

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solutions Asia Pacific

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Canvas/metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (etc…safety shoe)

Job Scope

# Lift up machine using jack

# Insert rollers under machine base

# Shift out machine from old location

# Transport machine to new location within production area

# Position machine at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175