# METHOD OF STATEMENT

Date: 28th Mar 2023

Description of work

To relocate 1 unit container bay from existing location to outside of factory premise

Spec

1 unit Container Test Bay

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

28/03/23

Time of Operation

12:00 pm

Location of Operation

#### 7 Loyang Way

NOV Oilwell Varco

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# To jack up container bay to 3” high

# Insert rollers under container bed

# Shift out from existing location

# Transport to outside factory premise

# Position at the new designated location

# Unload from rollers and let it sit on ground level

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani / Kevin Wong

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175