# METHOD OF STATEMENT

Date: 05th Jan 2021

Description of work

To relocate 6 unit machine from existing location to another location within factory premise

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/01/2021

Time of Operation

09:45 am

Location of Operation

#### 7 Loyang Way

NOV Oilwell Varco

Equipment Use

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Metal plate (for epoxy floor protection)

Obstacles

Ensure no obstacles around operation area and route to machine new location

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are equip with standard PPE (safety shoe)

Job Scope

# Lift up machine to 3” high

# Insert rollers/pallet truck under machine bed

# Shift out from existing location

# Transport to new location within factory premise

# Position machine at the new designated location

# Unload from rollers/pallet truck and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175